SECTION 08 71 00.18

WEATHERSTRIPS, THESHOLDS AND DOOR BOTTOMS

Display hidden notes to specifier. (Don't know how? Click Here)

** NOTE TO SPECIFIER ** Reese Enterprises, Inc.; weatherstrips, door thresholds and door bottoms.
This section is based on the products of Reese Enterprises, Inc., which is located at:
16350 Asher Ave.
P. O. Box 459
Rosemount, MN 55068-0459
Toll Free: 800-328-0953
Phone: 651-423-1126
Fax: 651-423-2662
Fax: 800-334-8823
Email: info@reeseusa.com
Web: www.reeseusa.com
click Herefor additional information
Reese Enterprises, Inc. has more than 80 years of experience in manufacturing weatherstrips and thresholds along with many other door and floor products. During this time, we have invested time and money into product improvements, product development, research and testing to continue to offer the highest quality products to our customers. Our highest valued asset, however, is you - our customer. We are here to offer you quality products, superior customer service, and on time delivery.
Reese Enterprises, Inc. offers the finest in weatherstrip, thresholds and entrance protection systems.

- GENERAL

SECTION INCLUDES

Weatherstrips, thresholds, door bottoms and auxiliary products.

RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

Section 08 71 53 - Security Door Hardware.

REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

ASTM International (ASTM):

ASTM D2287: Standard Specification for Nonrigid Vinyl Chloride Polymer and Copolymer Molding and Extrusion Compounds.

ASTM E152: Methods of Fire Tests of Door Assemblies.

National Fire Protection Association (NFPA): NFPA 252 - Standard Methods of Fire Test for Door Assemblies.

Underwriter's Laboratory (UL):

UL 10B - Standard for Fire Tests of Door Assemblies.

UL 10C - Standard for Positive Pressure Fire Tests of Door Assemblies.

** NOTE TO SPECIFIER ** Delete Article for Performance Requirements below, or individual paragraphs if not required.

PERFORMANCE REQUIREMENTS

** NOTE TO SPECIFIER ** Retain paragraph and subparagraph below when sound-control door assemblies are located in a fire-resistance-rated wall or partition. Verify testing requirements of authorities having jurisdiction. Note that closer pressures required to cause acoustic gaskets to compress adequately to latch door may require spring-type closer. Consult with manufacturer and requirements of authorities having jurisdiction.

Compliance for Fire-Rated Sound-Control Door and Frame Assemblies:

NFPA 80 compliant, listed and labeled by a testing and inspection agency acceptable to authorities having jurisdiction

Test at equalized pressure per UL 10C.

** NOTE TO SPECIFIER ** Edit accessibility requirements paragraph below to meet project requirements. Note that sound gasketing and threshold design required to achieve acoustical performance may require opening and closing forces in excess of that allowed under accessibility requirements. Consult manufacturer and requirements of authorities having jurisdiction.

SUBMITTALS

Submit under provisions of Section 01 30 00 - Administrative Requirements.

Product Data: Manufacturer's data sheets on each product to be used, including:

Preparation instructions and recommendations.

Maintenance instructions and recommendations.

QUALITY ASSURANCE

Manufacturer Qualifications: The manufacturer of products specified herein shall have a minimum 5 years of experience in the fabrication and installation of similar products.

Installer Qualifications: The installer shall have a minimum 2 years of experience in the installation of products similar to those specified herein.

DELIVERY, STORAGE, AND HANDLING

Deliver, store and handle materials and products in strict compliance with manufacturer's instructions and recommendations.

Store materials within absolute limits for temperature and humidity recommended by manufacturer. Protect from damage.

PROJECT CONDITIONS

Environmental Limitations: Do not deliver or install products until spaces are enclosed and weather tight, wet work in spaces is complete and dry, HVAC system is operating and maintaining ambient temperature at occupancy levels during the remainder of the construction period.

Field Measurements: Verify field measurements as indicated on Shop Drawings. Where measurements are not possible, provide control dimensions and templates.

Coordinate installation and location of blocking and supports as required.

Verify openings, clearances, storage requirements and other dimensions relevant to the installation and final application.

WARRANTY

Provide manufacturer's standard limited warranty for products. All Reese products are guaranteed for five years against defects in material or workmanship.

- PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Reese Enterprises, Inc., which is located at: 16350 Asher Ave. P. O. Box 459; Rosemount, MN 55068-0459; Toll Free Tel: 800-328-0953; Tel: 651-423-1126; Fax: 651-423-2662; Email:request info (info@reeseusa.com); Web:www.reeseusa.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

Substitutions: Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

** NOTE TO SPECIFIER ** Delete Article for thresholds and auxiliary products if not required.

** NOTE TO SPECIFIER ** For most projects, products in this section can/will be specified entirely on door hardware schedules and/or Drawings. If this is the case, generally retain only first paragraph and select options therein. Follow instructions in notes throughout this Section to avoid redundancy issues.

** NOTE TO SPECIFIER ** Delete Article for weatherstripping and auxiliary products if not required.

WEATHERSTRIPPING AND AUXILIARY PRODUCTS

** NOTE TO SPECIFIER ** For most projects, products in this section can/will be specified entirely on door hardware schedules and/or Drawings. If this is the case, generally retain only first paragraph and select options therein. Follow instructions in notes throughout this Section to avoid redundancy issues.

Weatherstrips and Auxiliary Products: Including but not limited to products as listed below as manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER ** Fill in blank, duplicate or delete. If filling in blank, ALSO retain paragraph(s) below for corresponding product(s), selecting options/attributes therein. The first two options are suitable for very small projects only. Retain one of first four subparagraphs as applicable to project type/phase.

Product: Reese model number_____DS75_______, quantities, fasteners, lengths and locations as indicated on Drawings with attributes as specified below in this Article.

** NOTE TO SPECIFIER ** Ordering fasteners only won't suffice for drilling. All aluminum weatherstrip comes standard with slotted holes; exception of DS69, 669 and 961. Delete two of three options.

Factory Drilling: As scheduled and indicated on Drawings.

Factory Drilling: None, provide undrilled.

Factory Drilling: Architect shall provide manufacturer with templates for factory drilled holes. Provide undrilled if Architect does not supply template.

** NOTE TO SPECIFIER ** Delete product types listed below not required for project.

Astragals.

Door jamb weatherstrips.

Vinyl and pile weatherstrips.

** NOTE TO SPECIFIER ** IF, in paragraph above, one of the first two options for ' Product...' is selected, THEN, in paragraphs below, select options/attributes with multiple ' choices' as applicable to project type/phase; filling in blanks as applicable. This is only suitable for very small projects in which Architect prefers to specify products herein, instead of on schedules and/or the drawings. IF, in paragraph above, the third or fourth options (more common) are selected, THEN Option 1) Delete remaining paragraphs in this Article altogether or 2) Within remaining paragraphs in this Article, DELETE all subparagraphs for product options/attributes with MULTIPLE ' choices'; retain only subparagraphs for ' fixed' attributes at top.
** NOTE TO SPECIFIER ** For heavy duty use, specify 59, 99, 399, 499 or 599 which feature a captive adjustment screw drilled and tapped into the solid bar; and one side of each piece mitered if ordered in sets. Where adjustable door stops are applied on top of existing stops, it is recommended that locks with 3-3/4 inch (95 mm) backsets be used rather than locks with a 2-3/4 inch (70 mm) backset. This allows adequate space between the door knob and the weatherstrip. Delete if not required.

Astragals: As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER ** Small projects only: Fill in blank with model number AND then select from attributes listed with multiple ' choices'; duplicate or delete as applicable. Retain one of first two options.

Model Number: Reese Model ____DS75___________.

** NOTE TO SPECIFIER ** If option below is selected, delete ALL remaining subparagraphs.

When astragals used on double doors, two lengths must be ordered, one for each door.

** NOTE TO SPECIFIER ** All models not available with all description listed below. Consult with manufacturer's website, literature or reps for details. Delete options for descriptions not required.

Description: Multi-component astragals for double doors.

** NOTE TO SPECIFIER ** All models not available in all materials listed below. Consult with manufacturer's website, literature or representatives for details. Delete options for materials not required.

Materials: Aluminum, alloy 6063-T5, mill finish.

Materials: Aluminum, alloy 6063-T5, clear anodized finish.

Materials: Aluminum, alloy 6063-T5, gold anodized finish.

Materials: Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER ** All models not available with all edge seals listed below. Consult with manufacturer's website, literature or reps for details. Delete options for edge seals not required.

Edge Seals: Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Edge Seals: Vinyl, ASTM D2287 and CS230-60 compliant, dark brown color.

** NOTE TO SPECIFIER ** Provides a 98 percent effective air tight seal to reduce heating/cooling. Helps protect against penetration of smoke, fumes, insects, rodents, snow, light, rain and wind. Remains flexible at -70 deg F (-57 deg C). Easy installation/adjustment. Most sizes come with factory drilled holes.

Door Jamb Weatherstrips: As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER ** Small projects only: Fill in blank with model number AND then select from attributes listed with multiple ' choices'; duplicate or delete as applicable. Retain one of first two options.

Model Number: Reese Model ____DS75___________.

** NOTE TO SPECIFIER ** If option below is selected, delete ALL remaining subparagraphs.

** NOTE TO SPECIFIER ** All models not available in all materials listed below. Consult with manufacturer's website, literature or representatives for details. Delete options for materials not required.

Materials: Aluminum, alloy 6063-T5, mill finish.

Materials: Aluminum, alloy 6063-T5, clear anodized finish.

Materials: Aluminum, alloy 6063-T5, gold anodized finish.

Materials: Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER ** All models not available with all edge seals listed below. Consult with manufacturer's website, literature or reps for details. Delete options for edge seals not required.

Edge Seals: Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Edge Seals: Vinyl, ASTM D2287 and CS230-60 compliant, dark brown color.

Vinyl and Pile Weatherstrips: As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER ** Small projects only: Fill in blank with model number AND then select from attributes listed with multiple ' choices'; duplicate or delete as applicable. Retain one of first two options.

Model Number: Reese Model ____DS75___________.

** NOTE TO SPECIFIER ** If option below is selected, delete ALL remaining subparagraphs.

** NOTE TO SPECIFIER ** All models not available in all materials listed below. Consult with manufacturer's website, literature or representatives for details. Delete options for materials not required.

Materials: Aluminum, alloy 6063-T5, mill finish.

Materials: Aluminum, alloy 6063-T5, clear anodized finish.

Materials: Aluminum, alloy 6063-T5, gold anodized finish.

Materials: Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER ** All models not available with all edge seals listed below. Consult with manufacturer's website, literature or reps for details. Delete options for edge seals not required.

Edge Seals: Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Edge Seals: Vinyl, ASTM D2287 and CS230-60 compliant, dark brown color.

- EXECUTION

EXAMINATION AND PREPARATION

Verify that field measurements, surfaces, substrates and conditions are as required, and ready to receive Work.

Prepare substrates using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

Do not proceed with installation until substrates have been properly prepared and deviations from manufacturer's recommended tolerances are corrected. Commencement of installation constitutes acceptance of conditions.

If preparation is the responsibility of another installer, notify Architect in writing of deviations from manufacturer's recommended installation tolerances and conditions

INSTALLATION

Install units plumb, square, in proper alignment with adjacent construction and secured to substrate, within manufacturer's recommended tolerances. Comply with manufacturer's installation instructions and approved submittals.

ADJUSTING

Prior to installation, adjust components to within tolerances recommended by manufacturer.

Adjust units and hardware so doors operate smoothly close with uniform frame alignment and seal compression.

CLEANING AND PROTECTION

Clean exposed surfaces. Touch up, refinish, or replace damaged components in a manner acceptable to Architect.

Protect installed products until completion of project.

Repair or replace defective work as directed by Architect upon inspection.

END OF SECTION