SECTION 08720 (08 71 00.18)

WEATHERSTRIPS, THESHOLDS AND DOOR BOTTOMS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2016-2020 ARCAT Inc. - All rights reserved

** NOTE TO SPECIFIER **  Reese Enterprises, Inc.; weatherstrips, door thresholds and door bottoms.

This section is based on the products of Reese Enterprises, Inc., which is located at:

16350 Asher Ave.

P. O. Box 459

Rosemount, MN 55068-0459

Toll Free: 800-328-0953

Phone: 651-423-1126

Fax: 651-423-2662

Fax: 800-334-8823

Email:  info@reeseusa.com

Web: www.reeseusa.com

{click  Here} for additional information

Reese Enterprises, Inc. has more than 80 years of experience in manufacturing weatherstrips and thresholds along with many other door and floor products. During this time, we have invested time and money into product improvements, product development, research and testing to continue to offer the highest quality products to our customers.  Our highest valued asset, however, is you – our customer. We are here to offer you quality products, superior customer service, and on time delivery.

Reese Enterprises, Inc. offers the finest in weatherstrips, thresholds and entrance protection systems.

GENERAL

SECTION INCLUDES

Weatherstrips, thresholds, door bottoms and auxiliary products.

RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

Section 08710 - Door Hardware for door hardware not specified in this Section.

REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

American National Standards Institute (ANSI):  ANSI 117.1 - Standard for Accessible and Usable Buildings and Facilities.

ASTM International (ASTM):

ASTM D2287:  Standard Specification for Nonrigid Vinyl Chloride Polymer and Copolymer Molding and Extrusion Compounds.

ASTM E 90:  Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements.

ASTM E152:  Methods of Fire Tests of Door Assemblies.

ASTM E283:  Standard Test Method for Determining Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen.

National Fire Protection Association (NFPA):  NFPA 252 - Standard Methods of Fire Test for Door Assemblies.

Underwriter's Laboratory (UL):

UL 10B - Standard for Fire Tests of Door Assemblies.

UL 10C - Standard for Positive Pressure Fire Tests of Door Assemblies.

UL 1784 - Standard for Air Leakage Tests of Door Assemblies and Other Opening Protectives.

** NOTE TO SPECIFIER **  Delete Article for Performance Requirements below, or individual paragraphs if not required.

PERFORMANCE REQUIREMENTS

Sound Rating:  Provide products compatible with assemblies for sound-control doors identical to assemblies tested by an independent testing agency per ASTM E90 with the specified minimum certified STC rating per ASTM E413 for the configurations indicated.

** NOTE TO SPECIFIER **  Retain paragraph and subparagraph below when sound-control door assemblies are located in a fire-resistance-rated wall or partition. Verify testing requirements of authorities having jurisdiction. Note that closer pressures required to cause acoustic gaskets to compress adequately to latch door may require spring-type closer. Consult with manufacturer and requirements of authorities having jurisdiction.

Compliance for Fire-Rated Sound-Control Door and Frame Assemblies:

NFPA 80 compliant, listed and labeled by a testing and inspection agency acceptable to authorities having jurisdiction

Test at equalized pressure per UL 10C.

** NOTE TO SPECIFIER **  Edit accessibility requirements paragraph below to meet project requirements.  Note that sound gasketing and threshold design required to achieve acoustical performance may require opening and closing forces in excess of that allowed under accessibility requirements.  Consult manufacturer and requirements of authorities having jurisdiction.

Compliance with Accessibility Requirements:

** NOTE TO SPECIFIER **  Insert local requirements in blank below, or delete line as applicable.  Delete compliance standards listed below not required for project.

_________________

Accessibility Guidelines for Buildings and Facilities (ADAAG).

ANSI A117.1.

SUBMITTALS

Submit under provisions of Section 01300.

Product Data:  Manufacturer's data sheets on each product to be used, including:

Preparation instructions and recommendations.

Maintenance instructions and recommendations.

QUALITY ASSURANCE

Manufacturer Qualifications:  The manufacturer of products specified herein shall have a minimum 5 years of experience in the fabrication and installation of similar products.

Installer Qualifications:  The installer shall have a minimum 2 years of experience in the installation of products similar to those specified herein.

DELIVERY, STORAGE, AND HANDLING

Deliver, store and handle materials and products in strict compliance with manufacturer's instructions and recommendations.

Store materials within absolute limits for temperature and humidity recommended by manufacturer.  Protect from damage.

PROJECT CONDITIONS

Environmental Limitations:  Do not deliver or install products until spaces are enclosed and weather tight, wet work in spaces is complete and dry, HVAC system is operating and maintaining ambient temperature at occupancy levels during the remainder of the construction period.

Field Measurements:  Verify field measurements as indicated on Shop Drawings.  Where measurements are not possible, provide control dimensions and templates.

Coordinate installation and location of blocking and supports as required.

Verify openings, clearances, storage requirements and other dimensions relevant to the installation and final application.

WARRANTY

Provide manufacturer's standard limited warranty for products.  All Reese products are guaranteed for five years against defects in material or workmanship.

PRODUCTS

MANUFACTURERS

Acceptable Manufacturer:  Reese Enterprises, Inc.; 16350 Asher Ave, P. O. Box 459, Rosemount, MN 55068-0459.  ASD.  Toll Free: 800-328-0953.  Phone: 651-423-1126.  Fax: 651-423-2662.  Fax: 800-334-8823.  Email:  info@reeseusa.com.  Web:  www.reeseusa.com.

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

Substitutions: Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01600.

** NOTE TO SPECIFIER **  Delete Article for thresholds and auxiliary products if not required.

THRESHOLDS AND AUXILIARY PRODUCTS

** NOTE TO SPECIFIER **  For most projects, products in this section can/will be specified entirely on door hardware schedules and/or Drawings.  If this is the case, generally retain only first paragraph and select options therein.  Follow instructions in notes throughout this Section to avoid redundancy issues.

Thresholds and Auxiliary Products:  Including but not limited to products as listed below as manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Fill in blank, duplicate or delete. If filling in blank, ALSO retain paragraph(s) below for corresponding product(s), selecting options/attributes therein.  The first two options are suitable for very small projects only.  Retain one of first four subparagraphs as applicable to project type/phase.

Product:  Reese model number_______________, quantities, fasteners, lengths and locations as indicated on Drawings with attributes as specified below in this Article.

Products and Attributes:  Provide models in quantities, lengths and locations with fasteners as indicated on Drawings with attributes as specified below in this Article.

** NOTE TO SPECIFIER **  For most projects, large or small:  Retain one of the two following options and delete the two options above.  Then: Option 1) Delete remaining paragraphs in this Article altogether. or 2) Within remaining paragraphs in this Article, DELETE all subparagraphs for product options/attributes with MULTIPLE ‘choices’; retaining only subparagraphs for ‘fixed’ attributes at top end of paragraphs.

Products and Attributes:  Provide models in quantities, lengths and locations with attributes and fasteners as scheduled and indicated on Drawings.

Products and Attributes:  Provide models in quantities, lengths and locations with fasteners as selected by Architect.

** NOTE TO SPECIFIER **  Ordering fasteners only won’t suffice for drilling.  Delete two of three options.

Factory Drilling:  As scheduled and indicated on Drawings.

Factory Drilling:  None, provide undrilled.

Factory Drilling:  Architect shall provide manufacturer with templates for factory drilled holes.  Provide undrilled if Architect does not supply template.

** NOTE TO SPECIFIER **  Delete product types listed below not required for project.

Assembly components.

Beveled aluminum plates.

Carpet separator thresholds.

Carpet separator thresholds. Semi-rigid PVC.

Hospitality, Carpet and tile transition thresholds. Semi-rigid PVC.

Combination thresholds.

Combination thresholds. Carpet to tile. Semi-rigid PVC.

Communicating door flat threshold. Semi-rigid PVC.

Corrugated flat plates.

Fiberglass thresholds.

Half saddle thresholds.

Interlock/waterproof thresholds.

Offset saddle thresholds.

Panic type thresholds.

Metal ramps.

Rubber ramps.

Semi-rigid PVC carpet to ground ramps.

Semi-rigid PVC carpet to VCT ramps.

Saddle thresholds.

Stainless steel thresholds.

Thermal break thresholds.

Tile separator, handicap accessible thresholds. ADA compliant. Semi-rigid PVC.

Vinyl top thresholds.

** NOTE TO SPECIFIER **  IF, in paragraph above, the first option for ‘Product...’ is selected, THEN, in paragraphs below, select options/attributes with multiple ‘choices’ as applicable to project type/phase; filling in blanks as applicable.  This is only suitable for very small projects in which Architect prefers to specify products herein, instead of on schedules and/or the drawings. IF, in paragraph above, the second or third options (more common) are selected, THEN Option 1) Delete remaining paragraphs in this Article altogether or 2) Within remaining paragraphs in this Article, DELETE all subparagraphs for product options/attributes with MULTIPLE ‘choices’; retain only subparagraphs for ‘fixed’ attributes at top.

Assembly Components:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL subparagraphs for product attributes that have MULITIPLE ‘choices’; retaining only subparagraph(s) for ‘Materials’.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Semi-rigid PVC. UL10B Fire tested.

Color:  Black

Color:  Grey stone.

Color: Tan stone.

** NOTE TO SPECIFIER **  All models not available with all or any inserts listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for inserts not required.

Inserts:  As scheduled and indicated on Drawings.

Inserts:  Silicone materials, black color.

Inserts:  Polyurethane, elastomeric thermoplastic, black color.

Inserts:  Polyprene, proprietary Reese thermoplastic rubber compound, black color.

Inserts:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Inserts:  None.

Beveled Aluminum Plates:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL subparagraphs for product attributes that have MULITIPLE ‘choices’; retaining only subparagraph(s) for ‘Materials’.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Minimum Plate Width Available:  2 inches (51 mm).

Maximum Plate Width Available:  24 inches (610 mm).

** NOTE TO SPECIFIER **  Fill in blank with width (for small projects only).  Delete one of two options.

Plate Width:  _______________.

Plate Width:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all thicknesses listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for thickness not required.

Thickness:  As scheduled and indicated on Drawings.

Thickness:  1/8 inch (3.2 mm).

Thickness:  1/4 inch (6.4 mm).

Carpet Separator Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Semi-rigid PVC. UL10B Fire tested.

Color:  Black

Color:  Grey stone.

Color: Tan stone.

** NOTE TO SPECIFIER **  All models not available in all depths listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for depth not required.

Depth:  As scheduled and indicated on Drawings.

Depth:  As scheduled and indicated on Drawings.

Depth:  7/16 inch (11.1 mm).

Depth:  1/2 inch (12.7 mm).

Depth:  23/32 inch (18.2 mm)

Combination Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Depth:  5/8 inch (15.9 mm).

Depth:  23/32 inch (18.2 mm)

** NOTE TO SPECIFIER **  Delete options for material not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Semi-rigid PVC. UL10B Fire tested.

Color:  Black

Color:  Grey stone.

Color: Tan stone.

** NOTE TO SPECIFIER **  All models not available in all widths listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for width not required.

Width:  As scheduled and indicated on Drawings.

Width:  3 inches (76 mm).

Width:  7-1/2 inches (191 mm).

** NOTE TO SPECIFIER **  All models not available with all or any inserts listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for inserts not required.

Inserts:  As scheduled and indicated on Drawings.

Inserts:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Inserts:  None.

Corrugated Flat Plates:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL subparagraphs for product attributes that have MULITIPLE ‘choices’; retaining only subparagraph(s) for ‘fixed’ attributes, above ‘choices’.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Thickness:  1/4 inch (6.4 mm).

Minimum Plate Width:  2 inches (51 mm).

Maximum Plate Width:  10 inches (254 mm).

Plate Width:  _______________.

Plate Width:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Architectural brass, alloy 385, mill finish.

** NOTE TO SPECIFIER **  Fill in blank with width (for small projects only).  Delete one of two options.

Fiberglass Thresholds:

Model Number:  Reese Model FBR-555 as manufactured by Reese Enterprises, Inc.

Materials:  Fiberglass.

Width:  5-1/2 inches (140 mm).

Depth:  1/2 inch (12.7 mm).

Half Saddle Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  Architectural brass is available ONLY with S239 and S245 series.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Architectural brass, alloy 385, mill finish.

** NOTE TO SPECIFIER **  All models not available with insert listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for inserts not required.

Inserts:  As scheduled and indicated on Drawings.

Inserts:  Silicone materials, black color.

Inserts:  None.

Interlock/Waterproof Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  Architectural brass is only available with T130 series.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for material not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Architectural brass, alloy 385, mill finish.

** NOTE TO SPECIFIER **  All models not available with all or any inserts listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for inserts not required.

Inserts:  As scheduled and indicated on Drawings.

Inserts:  Polyurethane, elastomeric thermoplastic, black color.

Inserts:  Polyprene, proprietary Reese thermoplastic rubber compound, black color.

Inserts:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Inserts:  Pile material, grey color.

Inserts:  None.

** NOTE TO SPECIFIER **  For applications requiring a threshold and a hook, both parts must be specified on your order.  Not applicable to all models.  Delete options for hooks not required.

Hooks:  As scheduled and indicated on Drawings.

Hooks:  Rolled metal hooks, alloy 6063-T5, mill finish.

Hooks:  Rolled metal hooks, brass, alloy 385, mill finish.

Hooks:  Aluminum, alloy 6063-T5, clear anodized finish.

Hooks:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Hooks:  Aluminum, alloy 6063-T5, gold anodized finish.

Hooks:  None.

** NOTE TO SPECIFIER **  The following options are available only with T150A.  Delete if not required.

Pans:  Provide optional pans for each unit in patterns submitted by Architect.

Pans:  Provide optional pans for units as scheduled and indicated on Drawings.

Pans:  None.

Weep Holes:  Provide optional factory drilled weep holes in thresholds.

Weep Holes:  Provide optional factory drilled weep holes in as scheduled and indicated on Drawings.

Weep Holes:  None.

Offset Saddle Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Architectural brass, alloy 385, mill finish.

** NOTE TO SPECIFIER **  All models not available in all widths listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for widths not required.

Width:  As scheduled and indicated on Drawings.

Width:  5-1/2 inches (140 mm).

Width:  7 inches (178 mm).

** NOTE TO SPECIFIER **  All models not available with all depths/offsets listed below.  Consult with manufacturer’s website, literature or reps for details. Delete options for depths/offset not required.

Depth/Offset:  As scheduled and indicated on Drawings.

Depth/Offset:  3/4 inch (19.1 mm) deep, 1/2 inch (12.7 mm) offset.

Depth/Offset:  1/2 inch (12.7 mm) deep, 1/4 inch (6.4 mm) offset.

Panic Type Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Architectural brass, alloy 385, mill finish.

** NOTE TO SPECIFIER **  All models not available in all widths listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for widths not required.

Width:  As scheduled and indicated on Drawings.

Width:  3-11/16 inches (94 mm).

Width:  3-3/4 inches (95 mm).

Width:  5 inches (127 mm).

** NOTE TO SPECIFIER **  All models not available with all offsets listed below.  Consult with manufacturer’s website, literature or representatives for details. Delete options for offset not required.

Offset:  As scheduled and indicated on Drawings.

Offset:  1/2 inches (12.7 mm).

Offset:  None.

** NOTE TO SPECIFIER **  All models not available with all or any inserts listed below.  Consult with manufacturer’s website, literature or representatives for details.  Vinyl insert shipped if no insert material specified.  Delete options for inserts not required.

Inserts:  As scheduled and indicated on Drawings.

Inserts:  Polyurethane, elastomeric thermoplastic, black color.

Inserts:  Polyprene, proprietary Reese thermoplastic rubber compound, black color.

Inserts:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Inserts:  Pile material, grey color.

Inserts:  None.

** NOTE TO SPECIFIER **   Beveled rise slope no greater than 1:2; available in slope less than 1:12.

Metal Ramp Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  The EZ Edge Ramp is solid and durable, with no load weight limitations. Designed for the real world, these ramps are remarkably affordable and easy to install on almost any surface. The EZ Edge Ramp system is your best value for vertical rise modifications. Multiple piece ramps are factory fitted between pads.  Superior slip-resistant surface; will not freeze, crack or flake.

Rubber Ramp Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese EZ Ramp Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL subparagraphs for product attributes that have MULITIPLE ‘choices’; retaining only subparagraph(s) for ‘fixed’ attributes, above ‘choices’.

Model Numbers and Attributes:  Reese EZ Ramp as scheduled and indicated on Drawings.

Materials:  100 percent reclaimed, 20 mesh minimum, crumb rubber with urethane binders.

Construction:  Compression molded ramps and traction pads.

Maximum Hardness (Shore A):  65.

Flared Sides:  22 degree angle.

Edges:  3/16 inch (4.8 mm) rounded edges.

** NOTE TO SPECIFIER **  All models not available with all widths/depths listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for widths/depths not required.

Width/Depth:  As scheduled and indicated on Drawings.

Width/Depth:  8 inches (94 mm) wide, 7/8 inch (22.3 mm) deep.

Width/Depth:  12-1/4 inches (311 mm) wide, 1-1/4 inches (32 mm) deep.

Saddle Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

Materials:  Architectural brass, alloy 385, mill finish.

** NOTE TO SPECIFIER **  All models not available with all widths/depths listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for widths/depths not required.

Width/Depth:  As scheduled and indicated on Drawings.

Width/Depth:  3 inches (76 mm) wide, 3/8 inch (9.5 mm) deep.

Width/Depth:  4 inches (102 mm) wide, 1/2 inch (12.7 mm) deep.

Width/Depth:  4 inches (102 mm) wide, 5/8 inch (15.9 mm) deep.

Width/Depth:  5 inches (127 mm) wide, 1/2 inch (12.7 mm) deep.

Width/Depth:  5 inches (127 mm) wide, 5/8 inch (15.9 mm) deep.

Width/Depth:  6 inches (152 mm) wide, 1/2 inch (12.7 mm) deep.

Width/Depth:  7 inches (178 mm) wide, 1/2 inch (12.7 mm) deep.

Width/Depth:  8 inches (203 mm) wide, 1/2 inch (12.7 mm) deep.

Stainless Steel Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is used, delete ALL subparagraphs below ‘Materials’.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Materials: 10 gauge, stainless steel, alloy 304, mill finish.

** NOTE TO SPECIFIER **  All models not available with all widths/depths listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for widths/depths not required.

Width/Depth:  As scheduled and indicated on Drawings.

Width/Depth:  4 inches (102 mm) wide, 1/4 inch (6.4 mm) deep.

Width/Depth:  4 inches (102 mm) wide, 1/2 inch (12.7 mm) deep.

Width/Depth:  5 inches (127 mm) wide, 1/4 inch (6.4 mm) deep.

Width/Depth:  5 inches (127 mm) wide, 1/2 inch (12.7 mm) deep.

Thermal Break Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL subparagraphs for product attributes that have MULITIPLE ‘choices’; retaining only subparagraph(s) for ‘fixed’ attributes, above ‘choices’.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Splines for Thermal Break:  Flexible vinyl.

** NOTE TO SPECIFIER **  Delete options for types not required.

Types:  As scheduled and indicated on Drawings.

Type:  Thermal break, saddle thresholds.

Type:  Thermal break, offset thresholds.

Type:  Thermal break, panic thresholds.

Type:  Thermal break, offset panic thresholds.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  All models not available with all or any inserts listed below.  Consult with manufacturer’s website, literature or representatives for details.  Vinyl insert shipped if no insert material specified.  Delete options for inserts not required.

Inserts:  As scheduled and indicated on Drawings.

Inserts:  Neoprene material, black color.

Inserts:  Polyurethane, elastomeric thermoplastic, black color.

Inserts:  Polyprene, proprietary Reese thermoplastic rubber compound, black color.

Inserts:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Inserts:  Pile material, grey color.

Inserts:  None.

Vinyl Top Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  Models available in select materials. Delete options for material not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  Delete Article for weatherstripping and auxiliary products if not required.

AUXILIARY PRODUCTS FOR WEATHERSTRIPS AND THESHOLDS

Auxiliary Products for Use with Weatherstrips and Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Fill in blank, duplicate or delete. If filling in blank, ALSO retain paragraph(s) below for corresponding product(s), selecting options/attributes therein.  The first two options are suitable for very small projects only.  Retain one of first four subparagraphs as applicable to project type/phase.

Product:  Reese model number_______________, quantities, fasteners, lengths and locations as indicated on Drawings with attributes as specified below in this Article.

Products and Attributes:  Provide models in quantities, lengths and locations with fasteners as indicated on Drawings with attributes as specified below in this Article.

** NOTE TO SPECIFIER **  For most projects, large or small:  Retain one of the two following options and delete the two options above.  Then: Option 1) Delete remaining paragraphs in this Article altogether. or 2) Within remaining paragraphs in this Article, DELETE all subparagraphs for product options/attributes with MULTIPLE ‘choices’; retaining only subparagraphs for ‘fixed’ attributes at top end of paragraphs.

Products and Attributes:  Provide models in quantities, lengths and locations with attributes and fasteners as scheduled and indicated on Drawings.

Products and Attributes:  Provide models in quantities, lengths and locations with fasteners as selected by Architect.

** NOTE TO SPECIFIER **  Ordering fasteners only won’t suffice for drilling.  Delete two of three options.

Factory Drilling:  As scheduled and indicated on Drawings.

Factory Drilling:  None, provide undrilled.

Factory Drilling:  Architect shall provide manufacturer with templates for factory drilled holes.  Provide undrilled if Architect does not supply template.

** NOTE TO SPECIFIER **  Delete product types listed below not required for project.

Carpet/flooring separators.

Carpet/flooring transition edges.

Metal ramps

Aluminum ramps.

Surface applied and elevator stop strips.

** NOTE TO SPECIFIER **  IF, in paragraph above, one of the first two options for ‘Product...’ is selected, THEN, in paragraphs below, select options/attributes with multiple ‘choices’ as applicable to project type/phase; filling in blanks as applicable.  This is only suitable for very small projects in which Architect prefers to specify products herein, instead of on schedules and/or the drawings. IF, in paragraph above, the third or fourth options (more common) are selected, THEN Option 1) Delete remaining paragraphs in this Article altogether or 2) Within remaining paragraphs in this Article, DELETE all subparagraphs for product options/attributes with MULTIPLE ‘choices’; retain only subparagraphs for ‘fixed’ attributes at top.

Carpet/Flooring Separator Thresholds:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  All models not available in all depths listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for depth not required.

Depth:  As scheduled and indicated on Drawings.

Depth:  As scheduled and indicated on Drawings.

Depth:  7/16 inch (11.1 mm).

Depth:  1/2 inch (12.7 mm).

Carpet/Flooring Transition Edges:  Beveled aluminum plates as manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL subparagraphs for product attributes that have MULITIPLE ‘choices’; retaining only subparagraph(s) for ‘Materials’.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Minimum Plate Width Available:  2 inches (51 mm).

Maximum Plate Width Available:  24 inches (610 mm).

** NOTE TO SPECIFIER **  Fill in blank with width (for small projects only).  Delete one of two options.

Plate Width:  _______________.

Plate Width:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all thicknesses listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for thickness not required.

Thickness:  As scheduled and indicated on Drawings.

Thickness:  1/8 inch (3.2 mm).

Thickness:  1/4 inch (6.4 mm).

** NOTE TO SPECIFIER **   Beveled rise slope no greater than 1:2; available in slope less than 1:12.

Metal Ramps:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  The EZ Edge Ramp is solid and durable, with no load weight limitations. Designed for the real world, these ramps are remarkably affordable and easy to install on almost any surface. The EZ Edge Ramp system is your best value for vertical rise modifications. Multiple piece ramps are factory fitted between pads.  Superior slip-resistant surface; will not freeze, crack or flake.

Rubber Ramps:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese EZ Ramp Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL subparagraphs for product attributes that have MULITIPLE ‘choices’; retaining only subparagraph(s) for ‘fixed’ attributes, above ‘choices’.

Model Numbers and Attributes:  Reese EZ Ramp as scheduled and indicated on Drawings.

Materials:  100 percent reclaimed, 20 mesh minimum, crumb rubber with urethane binders.

Construction:  Compression molded ramps and traction pads.

Maximum Hardness (Shore A):  65.

Flared Sides:  22 degree angle.

Edges:  3/16 inch (4.8 mm) rounded edges.

** NOTE TO SPECIFIER **  All models not available with all widths/depths listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for widths/depths not required.

Width/Depth:  As scheduled and indicated on Drawings.

Width/Depth:  8 inches (94 mm) wide, 7/8 inch (22.3 mm) deep.

Width/Depth:  12-1/4 inches (311 mm) wide, 1-1/4 inches (32 mm) deep.

Surface Applied and Elevator Stop Strips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  All models not available with all or any inserts listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for inserts not required.

Inserts:  As scheduled and indicated on Drawings.

Inserts:  Polyprene, proprietary Reese thermoplastic rubber compound, black color.

Inserts:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Inserts:  None.

EXECUTION

EXAMINATION AND PREPARATION

Verify that field measurements, surfaces, substrates and conditions are as required, and ready to receive Work.

Prepare substrates using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

Do not proceed with installation until substrates have been properly prepared and deviations from manufacturer’s recommended tolerances are corrected. Commencement of installation constitutes acceptance of conditions.

If preparation is the responsibility of another installer, notify Architect in writing of deviations from manufacturer’s recommended installation tolerances and conditions

INSTALLATION

Install units plumb, square, in proper alignment with adjacent construction and secured to substrate, within manufacturer's recommended tolerances. Comply with manufacturer's installation instructions and approved submittals.

ADJUSTING

Prior to installation, adjust components to within tolerances recommended by manufacturer.

Adjust units and hardware so doors operate smoothly close with uniform frame alignment and seal compression.

CLEANING AND PROTECTION

Clean exposed surfaces. Touch up, refinish, or replace damaged components in a manner acceptable to Architect.

Protect installed products until completion of project.

Repair or replace defective work as directed by Architect upon inspection.

END OF SECTION