SECTION 08720 (08 71 00.18)

WEATHERSTRIPS, THESHOLDS AND DOOR BOTTOMS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2016-2020 ARCAT Inc. - All rights reserved

** NOTE TO SPECIFIER **  Reese Enterprises, Inc.; weatherstrips, door thresholds and door bottoms.

This section is based on the products of Reese Enterprises, Inc., which is located at:

16350 Asher Ave.

P. O. Box 459

Rosemount, MN 55068-0459

Toll Free: 800-328-0953

Phone: 651-423-1126

Fax: 651-423-2662

Fax: 800-334-8823

Email:  info@reeseusa.com

Web: www.reeseusa.com

{click  Here} for additional information

Reese Enterprises, Inc. has more than 80 years of experience in manufacturing weatherstrips and thresholds along with many other door and floor products. During this time, we have invested time and money into product improvements, product development, research and testing to continue to offer the highest quality products to our customers.  Our highest valued asset, however, is you – our customer. We are here to offer you quality products, superior customer service, and on time delivery.

Reese Enterprises, Inc. offers the finest in weatherstrips, thresholds and entrance protection systems.

GENERAL

SECTION INCLUDES

Weatherstrips, thresholds, door bottoms and auxiliary products.

RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

Section 08710 - Door Hardware for door hardware not specified in this Section.

REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

ASTM International (ASTM):

ASTM D2287:  Standard Specification for Nonrigid Vinyl Chloride Polymer and Copolymer Molding and Extrusion Compounds.

ASTM E 90:  Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements.

ASTM E152:  Methods of Fire Tests of Door Assemblies.

ASTM E283:  Standard Test Method for Determining Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen.

National Fire Protection Association (NFPA):  NFPA 252 - Standard Methods of Fire Test for Door Assemblies.

Underwriter's Laboratory (UL):

UL 10B - Standard for Fire Tests of Door Assemblies.

UL 10C - Standard for Positive Pressure Fire Tests of Door Assemblies.

UL 1784 - Standard for Air Leakage Tests of Door Assemblies and Other Opening Protectives.

** NOTE TO SPECIFIER **  Delete Article for Performance Requirements below, or individual paragraphs if not required.

PERFORMANCE REQUIREMENTS

Sound Rating:  Provide products compatible with assemblies for sound-control doors identical to assemblies tested by an independent testing agency per ASTM E90 with the specified minimum certified STC rating per ASTM E413 for the configurations indicated.

** NOTE TO SPECIFIER **  Retain paragraph and subparagraph below when sound-control door assemblies are located in a fire-resistance-rated wall or partition. Verify testing requirements of authorities having jurisdiction. Note that closer pressures required to cause acoustic gaskets to compress adequately to latch door may require spring-type closer. Consult with manufacturer and requirements of authorities having jurisdiction.

Compliance for Fire-Rated Sound-Control Door and Frame Assemblies:

NFPA 80 compliant, listed and labeled by a testing and inspection agency acceptable to authorities having jurisdiction

Test at equalized pressure per UL 10C.

** NOTE TO SPECIFIER **  Edit accessibility requirements paragraph below to meet project requirements.  Note that sound gasketing and threshold design required to achieve acoustical performance may require opening and closing forces in excess of that allowed under accessibility requirements.  Consult manufacturer and requirements of authorities having jurisdiction.

SUBMITTALS

Submit under provisions of Section 01300.

Product Data:  Manufacturer's data sheets on each product to be used, including:

Preparation instructions and recommendations.

Maintenance instructions and recommendations.

QUALITY ASSURANCE

Manufacturer Qualifications:  The manufacturer of products specified herein shall have a minimum 5 years of experience in the fabrication and installation of similar products.

Installer Qualifications:  The installer shall have a minimum 2 years of experience in the installation of products similar to those specified herein.

DELIVERY, STORAGE, AND HANDLING

Deliver, store and handle materials and products in strict compliance with manufacturer's instructions and recommendations.

Store materials within absolute limits for temperature and humidity recommended by manufacturer.  Protect from damage.

PROJECT CONDITIONS

Environmental Limitations:  Do not deliver or install products until spaces are enclosed and weather tight, wet work in spaces is complete and dry, HVAC system is operating and maintaining ambient temperature at occupancy levels during the remainder of the construction period.

Field Measurements:  Verify field measurements as indicated on Shop Drawings.  Where measurements are not possible, provide control dimensions and templates.

Coordinate installation and location of blocking and supports as required.

Verify openings, clearances, storage requirements and other dimensions relevant to the installation and final application.

WARRANTY

Provide manufacturer's standard limited warranty for products.  All Reese products are guaranteed for five years against defects in material or workmanship.

PRODUCTS

MANUFACTURERS

Acceptable Manufacturer:  Reese Enterprises, Inc.; 16350 Asher Ave, P. O. Box 459, Rosemount, MN 55068-0459.  ASD.  Toll Free: 800-328-0953.  Phone: 651-423-1126.  Fax: 651-423-2662.  Fax: 800-334-8823.  Email:  info@reeseusa.com.  Web:  www.reeseusa.com.

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

Substitutions: Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01600.

** NOTE TO SPECIFIER **  Delete Article for thresholds and auxiliary products if not required.

WEATHERSTRIPPING AND AUXILIARY PRODUCTS

** NOTE TO SPECIFIER **  For most projects, products in this section can/will be specified entirely on door hardware schedules and/or Drawings.  If this is the case, generally retain only first paragraph and select options therein.  Follow instructions in notes throughout this Section to avoid redundancy issues.

Weatherstrips and Auxiliary Products:  Including but not limited to products as listed below as manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Fill in blank, duplicate or delete. If filling in blank, ALSO retain paragraph(s) below for corresponding product(s), selecting options/attributes therein.  The first two options are suitable for very small projects only.  Retain one of first four subparagraphs as applicable to project type/phase.

Product:  Reese model number_______________, quantities, fasteners, lengths and locations as indicated on Drawings with attributes as specified below in this Article.

Products and Attributes:  Provide models in quantities, lengths and locations with fasteners as indicated on Drawings with attributes as specified below in this Article.

** NOTE TO SPECIFIER **  For most projects, large or small:  Retain one of the two following options and delete the two options above.  Then: Option 1) Delete remaining paragraphs in this Article altogether. or 2) Within remaining paragraphs in this Article, DELETE all subparagraphs for product options/attributes with MULTIPLE ‘choices’; retaining only subparagraphs for ‘fixed’ attributes at top end of paragraphs.

Products and Attributes:  Provide models in quantities, lengths and locations with attributes and fasteners as scheduled and indicated on Drawings.

Products and Attributes:  Provide models in quantities, lengths and locations with fasteners as selected by Architect.

** NOTE TO SPECIFIER **  Ordering fasteners only won’t suffice for drilling.  All aluminum weatherstrip comes standard with slotted holes; exception of DS69, 669 and 961. Delete two of three options.

Factory Drilling:  As scheduled and indicated on Drawings.

Factory Drilling:  None, provide undrilled.

Factory Drilling:  Architect shall provide manufacturer with templates for factory drilled holes.  Provide undrilled if Architect does not supply template.

** NOTE TO SPECIFIER **  Delete product types listed below not required for project.

Adjustable weatherstrips.

Astragals.

Brush weatherstrips.

Door jamb weatherstrips.

Edge seal systems/smoke and draft weatherstrips.

Hardware compatible weatherstrips.

Interlocking weatherstrips.

Intumescent seals.

Magnetic weatherstrips.

Neoprene weatherstrips.

Polyprene weatherstrips.

Polyurethane weatherstrips.

Self-adhesive strips.

Silicone weatherstrips.

Smoke and draft weatherstrips.

Stainless Steel weatherstrips.

Vinyl and pile weatherstrips.

** NOTE TO SPECIFIER **  IF, in paragraph above, one of the first two options for ‘Product...’ is selected, THEN, in paragraphs below, select options/attributes with multiple ‘choices’ as applicable to project type/phase; filling in blanks as applicable.  This is only suitable for very small projects in which Architect prefers to specify products herein, instead of on schedules and/or the drawings. IF, in paragraph above, the third or fourth options (more common) are selected, THEN Option 1) Delete remaining paragraphs in this Article altogether or 2) Within remaining paragraphs in this Article, DELETE all subparagraphs for product options/attributes with MULTIPLE ‘choices’; retain only subparagraphs for ‘fixed’ attributes at top.

** NOTE TO SPECIFIER **  For heavy duty use, specify 59, 99, 399, 499 or 599 which feature a captive adjustment screw drilled and tapped into the solid bar; and one side of each piece mitered if ordered in sets.  Where adjustable door stops are applied on top of existing stops, it is recommended that locks with 3-3/4 inch (95 mm) backsets be used rather than locks with a 2-3/4 inch (70 mm) backset.  This allows adequate space between the door knob and the weatherstrip.  Delete if not required.

Adjustable Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Compliance:  Fire rated for use as perimeter seals only; shall not be used in lieu of an integral pressed steel stop on fire rated installations.

Mitered Sides:  One side of each piece mitered for sets.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  Delete options for maximum adjustment range not required.

Maximum Adjustment Range:  As scheduled and indicated on Drawings.

Maximum Adjustment Range:  1/4 inch (6.4 mm).

Maximum Adjustment Range:  7/16 inch (11.1 mm).

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Edge Seals:  As scheduled and indicated on Drawings.

Edge Seals:  Neoprene material, black color.

Edge Seals:  Silicone materials, black color.

Astragals:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

When astragals used on double doors, two lengths must be ordered, one for each door.

** NOTE TO SPECIFIER **  All models not available with all description listed below.  Consult with manufacturer’s website, literature or reps for details. Delete options for descriptions not required.

Description:  As scheduled and indicated on Drawings.

Description:  Single astragals.

Description:  Single, adjustable astragals.

Description:  Single astragals for double doors.

Description:  Multi-component astragals for double doors.

Description:  Multi-component interlocking astragals for double doors.

Description:  Multi-component astragals for double doors, with snap covers to conceal fasteners.

Description:  Multi-component adjustable astragals for double doors.

Description:  Multi-component, self adjusting magnetic astragals for double doors.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

Materials:  Steel, grey prime finish.

Materials:  Architectural brass, alloy 385, mill finish.

Materials:  Stainless steel, alloy 304, 10 gauge, mill finish.

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Edge Seals:  As scheduled and indicated on Drawings.

Edge Seals:  Neoprene material, black color.

Edge Seals:  Silicone materials, black color.

Edge Seals:  Polyurethane, elastomeric thermoplastic, black color.

Edge Seals:  Polyprene, proprietary Reese thermoplastic rubber compound, black color.

Edge Seals:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Edge Seals:  Nylon brush, grey color.

Edge Seals:  Pile material, grey color.

Edge Seals:  None.

** NOTE TO SPECIFIER **  Provides a 98 percent effective air tight seal to reduce heating/cooling. Helps protect against penetration of smoke, fumes, insects, rodents, snow, light, rain and wind. Remains flexible at -70 deg F (-57 deg C).  Easy installation/adjustment.  Most sizes come with factory drilled holes.

Brush Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

** NOTE TO SPECIFIER **  All models not available with all brush lengths listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for brush length not required.

Brush Length:  As scheduled and indicated on Drawings.

Brush Length:  9/32 inch (7.1 mm).

Brush Length:  3/8 inch (9.5 mm).

Brush Length:  15/16 inch (24 mm).

Brush Length:  1-3/16 inches (30 mm).

Brush Length:  1-9/16 inches (40 mm).

Brush Length:  1-5/8 inches (41 mm).

Brush Length:  1-3/4 inches (44 mm).

Brush Length:  2 inches (51 mm).

Brush Length:  2-1/2 inches (64 mm).

Brush Length:  3 inches (76 mm).

Brush Length:  4 inches (102 mm).

Door Jamb Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Architectural brass, alloy 385, mill finish.

Materials:  Stainless steel, alloy 304, 10 gauge, mill finish.

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Edge Seals:  As scheduled and indicated on Drawings.

Edge Seals:  Neoprene material, black color.

Edge Seals:  Nylon brush.

Edge Seals:  Silicone materials, black color.

Edge Seals:  Polyurethane, elastomeric thermoplastic, black color.

Edge Seals:  Polyprene, proprietary Reese thermoplastic rubber compound, black color.

Edge Seals:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Edge Seals:  Pile material, grey color.

Edge Seals:  None.

Combined Edge Seal Systems/Smoke and Draft Control Gasketing:  Reese 588 Series as manufactured by Reese Enterprises, Inc.

Compliance:

Tested to the requirement of UL 10C, ASTM E152, NFPA 252, UL 10B, UBC 7-2.

Required for Category ‘B’ fire-rated doors to meet positive pressure and ‘S’ label.  Category G listed and category H listed.

Approved for installation with:

Listed Wood Core Fire Doors with Wood or Steel Fire Door Frames rated 20 minutes (with hose stream).

Listed Mineral Core Fire Doors and Steel Fire Door Frames rated 45, 60, 90 minutes.

Description:  1/8 inch (3.2 mm) clearance required between door and frame.

Has fin on one side to help seal for smoke and draft control.

Expands and seals around door when heated, allows door to operate if needed..

Materials:  Graphite-based intumescent with a protective PVC covering.

Width:  5/8 inch (15.9 mm).

** NOTE TO SPECIFIER **  Delete option for color not required.

Color:  As scheduled and indicated on Drawings.

Color:  Brown 588D.

Color:  Charcoal 588CH.

Color:  White 588W.

** NOTE TO SPECIFIER **  Closers and holders mounted on weatherstrips. No coping! Saves time and money. Side pieces have slotted holes. Top piece is drilled at job for close fit.

Hardware Compatible Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Width:  1-1/2 inches (38 mm).

Depth:  1/4 inch (6.4 mm).

** NOTE TO SPECIFIER **  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Edge Seals:  As scheduled and indicated on Drawings.

Edge Seals:  Neoprene material, black color.

Edge Seals:  Nylon brush.

Edge Seals:  Silicone materials, black color.

Edge Seals:  Polyurethane, elastomeric thermoplastic, black color.

Edge Seals:  Polyprene, proprietary Reese thermoplastic rubber compound, black color.

Edge Seals:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Edge Seals:  Pile material, grey color.

Edge Seals:  None.

Interlocking Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Models _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  All models not available with insert listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for inserts not required.

Inserts:  As scheduled and indicated on Drawings.

Inserts:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Inserts:  None.

Brush Length:  4 inches (102 mm).

Magnetic Weatherstrips:  Reese 574 Series as manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Depth:  5/8 inch (15.9 mm).

Width:  1 inch (25 mm).

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish; 574C.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish; 574G.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish; 574D.

** NOTE TO SPECIFIER **  Delete options for length not required.

Length:  As scheduled and indicated on Drawings.

Length:  36 inches (914 mm).

Length:  48 inches (1219 mm).

Length:  84 inches (2137 mm).

Length:  96 inches (2438 mm).

Neoprene Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Performance:  Withstands temperatures from -40 deg F to 200 deg F (-40 deg C to 93 deg C).

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

Polyprene Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Wear Resistance:  More wear resistant than silicone rubber.

Depth:  1/4 inch (6.4 mm).

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Width:  As scheduled and indicated on Drawings.

Width:  7/8 inch (22.2 mm).

Width:  3/4 inch (19.1 mm).

** NOTE TO SPECIFIER **  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

Polyurethane Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Performance:

4 times the tear resistance compared to silicone rubber.

40 times the tensile strength compared to silicone rubber.

Nearly 100 times the abrasion resistance compared to silicone rubber.

Flexible to -58 deg F (-50 deg C).

Resistant to ultraviolet light, fungus, petroleum based solvents and Halon fire extinguishant.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

Self-Adhesive Strips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Compliance:

Meets UL 10B, UL 10C Category J and Category H.

Meets CANA4-S104.

Can use on S labeled doors 17L5 9903

Tape Width:  3/8 inch (9.5 mm).

Overall Width:  1/2 inch (12.7 mm).

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Polyurethane base, elastomeric thermoplastic, black color; and Polyprene fins, proprietary Reese thermoplastic rubber compound, black color.

Materials:  Neoprene material, black color.

Materials:  Silicone materials, black color.

Materials:  Polyurethane, elastomeric thermoplastic, black color.

Materials:  Polyprene, proprietary Reese thermoplastic rubber compound, black color.

** NOTE TO SPECIFIER **  All models not available with all colors listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for colors not required.

Color:  As scheduled and indicated on Drawings.

Color:  Black.

Color:  Charcoal.

Color:  White.

Silicone Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Performance:

Withstands temperatures from -75 deg F to 400 deg F (-59 deg C to 204 deg C).

Resistant to ultraviolet light, fungus, mildew and moisture.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

Stainless Steel Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  Stainless steel, alloy 304, 10 gauge, mill finish.

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Edge Seals:  As scheduled and indicated on Drawings.

Edge Seals:  Neoprene material, black color.

Edge Seals:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Vinyl And Pile Weatherstrips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Edge Seals:  As scheduled and indicated on Drawings.

Edge Seals:  Pile material, grey color.

Edge Seals:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

DOOR BOTTOMS AND AUXILIARY PRODUCTS

Auxiliary Products for Use with Door Bottoms:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Fill in blank, duplicate or delete. If filling in blank, ALSO retain paragraph(s) below for corresponding product(s), selecting options/attributes therein.  The first two options are suitable for very small projects only.  Retain one of first four subparagraphs as applicable to project type/phase.

Product:  Reese model number_______________, quantities, fasteners, lengths and locations as indicated on Drawings with attributes as specified below in this Article.

Products and Attributes:  Provide models in quantities, lengths and locations with fasteners as indicated on Drawings with attributes as specified below in this Article.

** NOTE TO SPECIFIER **  For most projects, large or small:  Retain one of the two following options and delete the two options above.  Then: Option 1) Delete remaining paragraphs in this Article altogether. or 2) Within remaining paragraphs in this Article, DELETE all subparagraphs for product options/attributes with MULTIPLE ‘choices’; retaining only subparagraphs for ‘fixed’ attributes at top end of paragraphs.

Products and Attributes:  Provide models in quantities, lengths and locations with attributes and fasteners as scheduled and indicated on Drawings.

Products and Attributes:  Provide models in quantities, lengths and locations with fasteners as selected by Architect.

** NOTE TO SPECIFIER **  Ordering fasteners only won’t suffice for drilling.  Delete two of three options.

Factory Drilling:  As scheduled and indicated on Drawings.

Factory Drilling:  None, provide undrilled.

Factory Drilling:  Architect shall provide manufacturer with templates for factory drilled holes.  Provide undrilled if Architect does not supply template.

** NOTE TO SPECIFIER **  Delete product types listed below not required for project.

Automatic door bottoms.

Door extenders.

Door shoes.

Door sweeps.

Hospitality Dark-AN Door Bottoms with Pressure Regulating Alternating Notches.

Door sweeps with nylon brushes.

Rain drips.

** NOTE TO SPECIFIER **  IF, in paragraph above, one of the first two options for ‘Product...’ is selected, THEN, in paragraphs below, select options/attributes with multiple ‘choices’ as applicable to project type/phase; filling in blanks as applicable.  This is only suitable for very small projects in which Architect prefers to specify products herein, instead of on schedules and/or the drawings. IF, in paragraph above, the third or fourth options (more common) are selected, THEN Option 1) Delete remaining paragraphs in this Article altogether or 2) Within remaining paragraphs in this Article, DELETE all subparagraphs for product options/attributes with MULTIPLE ‘choices’; retain only subparagraphs for ‘fixed’ attributes at top.

** NOTE TO SPECIFIER **  Automatic Door bottoms are not designed to seal directly on carpet. For best results door bottoms should seal on a solid surface. The solid surface should be positioned above the level of the floor covering on either side of the door.  Automatic door bottom sizes 18 inches to 24 inches (610 mm) cannot be trimmed. 36 inches (914 mm), 42 inches (1067 mm) and 48 inches (1219 mm) can be trimmed up to 3 inches (76 mm). Other sizes can be trimmed up to 1 inch (25 mm).  Part 370 can be trimmed up to 2 inches (51 mm) in all sizes.  When Reese No. 571 Automatic Door Bottom used in conjunction with Reese No. 599 Series Head and Jamb Seal, STC Rating of 46 can be achieved.  Sound transmission loss ratings up to STC 50 have been achieved by several door manufacturers using the Reese No. 371 Series door bottom in conjunction with their sound rated door systems.

Automatic Door Bottoms:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  Delete options for maximum adjustment range not required.

Maximum Drop:  As scheduled and indicated on Drawings.

Maximum Drop:  1/2 inch (12.7 mm).

Maximum Drop:  3/4 inch (19.1 mm).

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Edge Seals:  As scheduled and indicated on Drawings.

Edge Seals:  Neoprene material, black color.

Edge Seals:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Edge Seals:  Nylon brush.

Edge Seals:  Lead, for X-ray shielding.

Edge Seals:  None.

Door Extenders:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

Internal Width:  1-3/4 inches (44 mm).

Inserts:  Pile material, grey color.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Door Shoes:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Edge Seals:  As scheduled and indicated on Drawings.

Edge Seals:  Polyurethane smooth insert, elastomeric thermoplastic, black color.

Edge Seals:  Vinyl insert with fingers, ASTM D2287 and CS230-60 compliant, grey color.

Edge Seals:  Pile material, grey color.

Edge Seals:  None.

Door Sweeps:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

Materials:  Architectural brass, alloy 385, mill finish.

Materials:  Stainless steel, alloy 304, 10 gauge, mill finish.

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Edge Seals:  As scheduled and indicated on Drawings.

Edge Seals:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Edge Seals:  Nylon brush, black color.

Edge Seals: Polyurethane, black color.

Edge Seals: Neoprene, black color.

Hospitality Dark-AN Door Bottoms with Pressure Regulating Alternating Notches:  As manufactured by Reese Enterprises, Inc.

Full PVC triple-fin door bottom with flexible triple fins.

** NOTE TO SPECIFIER **  Optional. Delete if not required. Id required, add “-AN” as a suffix to the part number being ordered.

Alternating notches in fins for pressure regulation.

Model Number:  Reese Model ________.

Fire Rated:  UL 10B – fire tested.

Positive Pressure:  UL 10C Category J.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Rigid and Flexible PVC. Color:  Brown.

Edge Seals: Rigid and Flexible PVC, black color.

Door Sweeps with Nylon Brushes:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  As scheduled and indicated on Drawings.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

** NOTE TO SPECIFIER **  All models not available with all brush lengths listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for brush length not required.

Brush Length:  As scheduled and indicated on Drawings.

Brush Length:  9/32 inch (7.1 mm).

Brush Length:  3/8 inch (9.5 mm).

Brush Length:  15/16 inch (24 mm).

Brush Length:  1-3/16 inches (30 mm).

Brush Length:  1-9/16 inches (40 mm).

Brush Length:  1-5/8 inches (41 mm).

Brush Length:  1-3/4 inches (44 mm).

Brush Length:  2 inches (51 mm).

Brush Length:  2-1/2 inches (64 mm).

Brush Length:  3 inches (76 mm).

Brush Length:  4 inches (102 mm).

Rain Drips:  As manufactured by Reese Enterprises, Inc.

** NOTE TO SPECIFIER **  Small projects only:  Fill in blank with model number AND then select from attributes listed with multiple ‘choices’; duplicate or delete as applicable.  Retain one of first two options.

Model Number:  Reese Model _______________.

** NOTE TO SPECIFIER **  If option below is selected, delete ALL remaining subparagraphs.

Model Numbers and Attributes:  As scheduled and indicated on Drawings.

** NOTE TO SPECIFIER **  All models not available in all materials listed below.  Consult with manufacturer’s website, literature or representatives for details.  Delete options for materials not required.

Materials:  Aluminum, alloy 6063-T5, mill finish.

Materials:  Aluminum, alloy 6063-T5, clear anodized finish.

Materials:  Aluminum, alloy 6063-T5, gold anodized finish.

Materials:  Aluminum, alloy 6063-T5, dark bronze anodized finish.

Materials:  Aluminum, alloy 6063-T5, black anodized finish.

Materials:  Architectural brass, alloy 385, mill finish.

** NOTE TO SPECIFIER **  All models not available with all edge seals listed below.  Consult with manufacturer’s website, literature or reps for details.  Delete options for edge seals not required.

Edge Seals:  As scheduled and indicated on Drawings.

Edge Seals:  Vinyl, ASTM D2287 and CS230-60 compliant, grey color.

Edge Seals:  Nylon brush.

EXECUTION

EXAMINATION AND PREPARATION

Verify that field measurements, surfaces, substrates and conditions are as required, and ready to receive Work.

Prepare substrates using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

Do not proceed with installation until substrates have been properly prepared and deviations from manufacturer’s recommended tolerances are corrected. Commencement of installation constitutes acceptance of conditions.

If preparation is the responsibility of another installer, notify Architect in writing of deviations from manufacturer’s recommended installation tolerances and conditions

INSTALLATION

Install units plumb, square, in proper alignment with adjacent construction and secured to substrate, within manufacturer's recommended tolerances. Comply with manufacturer's installation instructions and approved submittals.

ADJUSTING

Prior to installation, adjust components to within tolerances recommended by manufacturer.

Adjust units and hardware so doors operate smoothly close with uniform frame alignment and seal compression.

CLEANING AND PROTECTION

Clean exposed surfaces. Touch up, refinish, or replace damaged components in a manner acceptable to Architect.

Protect installed products until completion of project.

Repair or replace defective work as directed by Architect upon inspection.

END OF SECTION